Rev.1.00

Mar.5,2021

Renesas RA6T1

IBIS Model Release Note

- Version　　　　Rev.1.0.0

- IBIS file

| File name | Conditions |  |  |
| --- | --- | --- | --- |
|  | TYP | MIN | MAX |
| ra6t1_vcc33_rev100.ibs | VCC=3.3V | VCC=3.0V | VCC=3.6V |
|  | Tj= 40℃ | Tj= 125℃ | Tj= -40℃ |
| ra6t1_vcc30_rev100.ibs | VCC=3.0V | VCC=2.7V | VCC=3.3V |
|  | Tj= 40℃ | Tj= 125℃ | Tj= -40℃ |

- Target device

All products in the RA6T1 are included. Select one of the components in the IBIS file indicated in the following table for each package.

| component name. | corresponding package |
| --- | --- |
| LQFP100 | 100-pin LQFP package |
| LQFP64 | 64-pin LQFP package |

Model Selector] Notes

Used to pick a [Model] from a list of [Model] for a pin which uses a programmable buffer. Select one of the models listed in the [Model Selector] according to the user’s MCU setting. Example:

[Pin] signal name model name

1  P400  iic0h_selector

2  P401  iic0h_selector

3  P402  io05_selector

:

:

[Model Selector]    iic0h_selector

iic0h_stdfm                 IIC buffer for STD/FM

iic0h_fmplus               IIC buffer for FM+

io5vtol02_drv_low      Drive Capability Low

io5vtol02_drv_middle Drive Capability Middle

io5vtol02_drv_high     Drive Capability High

:

:

Revision History

| Rev. | Date | Description |  |
| --- | --- | --- | --- |
|  |  | Page | Summary |
| 1.00 | Mar. 5,2021 | - | First Edition |