Test Procedure for ISL85412/85413EVAL1ZA Evaluation Board Revision A

INITIAL SET UP

- Set SW1 to the OFF position.

- Set SW2 to PFM.

- Connect an adjustable power supply capable of sourcing of at least 1A-50V to VIN and PGND1. Set to 0V before connection.

- Connect output load to VOUT and PGND2.  Set load to 0A.

- Use Digital Multi-Meters (DMMs) to monitor VOUT.

POWER UP

- Take the adjustable power supply that is connected to the VIN terminals and do the following:

- Adjust the VIN source from 0V to 5V.

- Take VOUT measurements using DMM. The readings should be: VOUT={3.270, 3.330}.

- The LED D1 should light up.

- Adjust the VIN source to 30V.

- Take VOUT measurements using DMM. The readings should be: VOUT1={3.270, 3.330}.

- Adjust VIN power supply to 5V and do the following:

- Increase output loadings of VOUT from 0A to 0.15A for ISL85412EVAL1Z, 0.3A for ISL85413EVAL1Z.

- Take VOUT{3.270, 3.330}. Measure input current (Iin).  Passing board must be less than 0.11A for ISL85412EVAL1Z, less than 0.25A for ISL85413EVAL1Z.

- Adjust VIN to 30V.

- Take VOUT {3.270, 3.330}. Measure the input current (Iin).  Passing board must be less than 0.03A for ISL85412EVAL1Z, less than 0.05A for ISL85413EVAL1Z.

- Decrease output loading to 0A.

- Adjust VIN to 12V.

- Measure the input current (Iin). Passing board must have less than 3.5mA.

- Toggle SW2 to PWM.

- Measure the input current (Iin). Passing board must have less than 1.5mA.

- Take VOUT measurement using DMM. The reading should be: VOUT={3.270, 3.330}.

- Toggle SW1 to ON. LED D1 should turn off.

- Toggle SW1 back to OFF. LED D1 should light up.

- Adjust VIN to 2.8V. The output should shut down.

- If Smoke, Fire, or Detonation occurs, then the board has failed test.

- Toggle SW1 to ON position.

- Power down VIN to 0V.

The board should be shipped with the present settings.