Preliminary only, subjected to change without notification.

# INTRODUCTION

The ISL97682 Evaluation Board (Rev. A) provides a complete testing platform for ISL97682, a two channel LED driver. Please refer to the product datasheet for detailed information including pinout, pin function description, electrical specifications, applications related information, etc.

# INSTRUCTION

Please follow the steps described below to start your evaluation.

- For both switch #1 and #2 (SW1 and SW2 shown on the board), set them to position 3 (left side).

- For enable control jumper, JP1, put the shunt to “ON” position (right side) to connect EN pin to VIN. When put the shunt to “OFF” position, it will disable the chip by pulling EN pin to ground.

- Connect JP14 so the VIN pin is connected to PVIN.

- For JP20, connect the shunt to the upper position.

- Connect WR and JP2-JP6.

- Apply 2V~5.5V PWM signal between PWMI pin and AGND.

- Apply 4.5V~26V between PVIN and PGND and the LEDs should be lit, and you can start the evaluation.

Note:

- In step #1 above, the SW1, SW2 position can be adjusted to different positions for different configurations, detail is provided below:

| SW1 position | SW2 position | Description |
| --- | --- | --- |
| 1 | 1 | LX switching frequency = 600KHz, phase shift between CH1-CH2 |
| 3 | 3 | LX switching frequency = 1MHz, phase shift between CH1-CH2 |
| 3 | 1 | LX switching frequency = 600KHz, no phase shift between CH1-CH2 |
| 1 | 3 | LX switching frequency = 1MHz, no phase shift between CH1-CH2 |

- The LED maximum DC current adjustment.

For each channel, the maximum DC current is set by resistance connected to RSET pin. The current for each channel can be calculated as

| I_LED(mA)=402/RSET(kΩ) | EQ. (1) |
| --- | --- |

On the board, a potentiometer R5 and a few other resistors are provided for easily adjust the LED maximum DC current; please refer to the attached evaluation board schematic for more details.

- LED dimming frequency and duty cycle.

- As mentioned in step #4 above, when shunt on JP20 is connected to the upper position, FPWM/DPWM pin is connected to VDC, the device enters direct PWM mode, which means both the LED dimming frequency and the duty cycle is synchronized with the external PWM signal applied on PWMI pin.

- When shunt on JP20 is connected to the lower position, FPWM/DPWM pin is connected to a resistor. Under such condition, the LED dimming frequency of the chip is programmed by the resistance connected on FPWM/DPWM pin following the equation of:

| FPWM(Hz)=12.4*10^7/R_FPWM(Ω) | EQ. (2) |
| --- | --- |

While the duty cycle is still modulated by the external PWM signal applied on PWMI pin. On board, a potentiometer R11 and a few other resistors are provided for easily adjust the LED dimming frequency under such configuration.

# EVALUATION BOARD SCHEMATIC

# PCB LAYOUT

Top Silkscreen Layer + Top Layer

Bottom Layer

# Bill of Materials (BOM)

| Designator | Part Type | Footprint | Part Manufacturer/Number |
| --- | --- | --- | --- |
| R1 | 150k | 603 | 1% SMD Resistor |
| R2 | 100k | 603 | General purpose |
| R3 | 358k | 603 |  |
| R4 | 10k | 603 |  |
| R5 | 1M | VRES |  |
| R6 | open |  |  |
| R7 | 38.3k | 603 |  |
| R8 | 5k | 603 |  |
| R9 | 10k | 603 |  |
| R10 | 0 | 603 |  |
| R11 | 100k | VRES |  |
| R12 | 0 | 603 |  |
| R13 | Open | 603 |  |
| L1 | 22uH |  | Cooper Bussmann, SD18-220 |
| D1 | NSR05F40NXT |  |  |
| C1 | 4.7uF/50V | 1210 | General purpose |
| C2 | 0.1uF/50V | 603 | Ceramic X5R/X7R capacitors |
| C3 | 390pF | 603 |  |
| C4 | 1uF/16V | 603 |  |
| C5 | 4.7uF/50V | 1210 | Murata, GRM32ER71H475KA88L |
| C6 | open | 1210 |  |
| C7 | Place Holder | 1210 | Not Populated |
| C8 | Place Holder | 1210 |  |
| C9 | 100pF/50V | 603 | General purpose |
| C10 | 3.6nF/50V | 603 | Ceramic X5R/X7R capacitors |
| C11 | 1nF/50V | 603 |  |
| C12 | 1nF/50V | 603 |  |
| C13 | 1nF/50V | 603 |  |
| C14 | 1nF/50V | 603 |  |
| C15 | 3.3nF | 603 |  |
| F1 | 2A Fuse | 1206 | Bel Fuse Inc, C1Q 2 |
| U1 |  | QFN16 3MM | Intersil, ISL97682/3/4 |
| JP2-JP19 | JUMPER-2PIN | JUMPER-2PIN | FCI |
| WR | JUMPER-2PIN | JUMPER-2PIN | 68000-236HLF-1x2 |
| JP1 | JUMPER-3PIN | JUMPER-3PIN | FCI |
| JP20 | JUMPER-3PIN | JUMPER-3PIN | 68000-236HLF-1x3 |
| LED1-12 LED25-36 | LED-SMT | LW_Y87C |  |
| TP1 | LX | TEST POINT | Keystone Electronics |
| TP2 | VOUT | TEST POINT | 5010 |
| TP3 | CH1 | TEST POINT |  |
| TP4 | CH2 | TEST POINT |  |
| TP5 | CH3 | TEST POINT |  |
| TP6 | CH4 | TEST POINT |  |
| TP7 | VDC | TEST POINT |  |
| TP8 | RSET | TEST POINT |  |
| TP9 | FSW/PHS | TEST POINT |  |
| P5 | AGND | TEST POINT | Keystone Electronics |
| P6 | AGND | TEST POINT | 5011 |
| P9 | PGND | TEST POINT |  |
| P1 | PVIN | POWERPOST | Mill Max |
| P2 | VIN | POWERPOST | 3156-1-00-00-00-00-08-0 |
| P3 | EN | POWERPOST |  |
| P4 | PWMI | POWERPOST |  |
| P7 | AGND | POWERPOST |  |
| P8 | AGND | POWERPOST |  |
| P10 | PGND | POWERPOST |  |
| SW2 | SPDT | SWITCH-SLIDE-SPDT | EAO |
| SW1 | SPDT | SWITCH-SLIDE-SPDT | 09.03201.02 |