### 8 HOUR TURN TIME REQUIRED FOR ALL PCB QUOTES

| DESIGN CONTACT FOR QUESTIONS: | X; MARK HINCHMAN 321/724-7248. Alan Erzinger / PHONE # 321-729-4915. |
| --- | --- |
| INTERSIL P/N: RoHS Compliant Required? | RTKA-BDGSTKEV1Z YES |
| # SOLDER SAMPLE REQUIRED | 0 |
| # REV # | A |
| ZIP FILE NAME: | RTKA-BDGSTKEV1Z |
| QTY REQUIRED: | 100 |
| TURN TIME: | 10 DAYS |
| NUMBER OF LAYERS: | 4 |
| # BOARD DIMENSIONS: X | 2.42 inches |
| Y | 0.76 inches |
| PANEL REQUIRED? ( 5 UP - 1/4” EDGE BREAKAWAY) | YES BREAKAWAY PANELS |
| THICKNESS: | .062 |
| COPPER WEIGHT: | 1 0Z |
| MATERIAL: | FR4 ROHS |
| TOTAL HOLE QTY: | 115 |
| LARGEST: | 91 mils |
| SMALLEST: | 10 mils |
| THRU HOLE: |  |
| SMD: |  |
| TOTAL NUMBER OF SMD PADS: | 298 |
| # TOP: | 191 |
| # BOTTOM: | 97 |
| #### TRACES & SPACES? |  |
| PLATING REQ’D ? |  |
| ## SMOBC? | NO |
| ## IMMERSION GOLD? | YES |
| ## HARD GOLD? | NO |
| # OTHER? |  |
| ARRAY FORMAT REQUIRED? | NO |
| ARRAY GERBER FILES SENT TO: |  |
| TEST REQUIRED | 100% TEST W/NETLIST PROVIDED (IPC 356) |
| CONTROLLED IMPEDANCE? | NO |
| ## VALUE? +/- PERCENT? | NO |
| DIELECTRIC CONSTANT | NO |
| MICROSTRIP LINE WIDTH? | NO |
| MICROSTRIP SPACING BETWEEN LINES | NO |
| DIELECTRIC THICKNESS BETWEEN GROUND AND MICROSTRIP | NO |
| TDR COUPON DATA REQUESTED? |  |
| # BLIND/BURIED VIAS? (If yes, what layers?) | NO |
| SILKSCREEN: (1 SIDED OR 2 SIDED) | 2 side |
| SOLDERMASK: | BLUE |
| COMMENTS: SEPECIAL INSTRUCTIONS | 45 DEGREE CORNERS. DIMENSIONS ON DRILL DRAWING |