# DA14580AT3B-P_VB

Board format: 34.3x42.38mm

Number of layers: 2

Silkscreen:  Top and Bottom

Total thickness:  1.6 mm

Material: FR-4

Finish: ENIG

Description of files,

| TOP.art | Top copper layer – outer |
| --- | --- |
| BOTTOM.art | Bottom copper layer – outer |
| SST.art | Silkscreen top layer |
| SSB.art | SilkScreen bottom layer |
| SMT.art | Soldermask top layer |
| SMB.art | Soldermask bottom layer |
| da14580a3db-p_vb-1-2.drl | Drill file – plated through holes |
| da14580a3db-p_vb-1-2 -np.drl | Drill file – non-plated through holes |
| Nc_param.txt | Drill file parameters |
| Art_param.txt | Gerber parameters |

Please panelize the PCBs in two rows of 5 PCBs and have a rim on two sides of the panel of 5mm.

Don’t make any mechanical connections within the red circles shown in the picture below as to not obstruct the edge connector. Mechanical connections should be at least 8mm away from the edge.

The best method to use to be able to break the PCBs out of the panel (v-score, milling, etc) is left at the discretion of the PCB manufacturer (please note the angled corners of the PCBs).

Please mill one edge of the PCB (card edge connector) on both sides (top and bottom) with a 30 degree angle and a width of 0.5mm.

The card edge connector does not require hard plated gold.